Middlesex University Motion Platform

Last update M Margolis Feb 3 2023

The design of the new motion system provides a large unencumbered top platform with much greater load capacity and versatility than the current chairs. The design is a modified Stewart platform where distances between attachment points is controlled by a form of slider crank mechanism actuated by pneumatic muscles. The upper platform size is 120x140cm. The maximum payload is 350Kg.

Motion platform (top platform not shown)

Muscles fully extended 						Muscles fully contracted

The lower attachment joints cans slide 200mm actuated by pneumatic muscles. The cranking action results in a range of 150mm of vertical movement. Six degrees of freedom of the platform are controlled by the relative position of each of the six lower attachment points.

Top view							Side View

Base with mount points for actuators			  Top frame with mount points

One pair actuator rails with carriages 				Actuator rod with ball joints

Component details:

Drive system: 	Festo pneumatic components as follows:

3473128 CPX-CEC-C1-V3 control blocks (CODESYS V3 OpenCAN)

195708 CPX-AB-8-KL-4POL manifold block (2 off)

542218 VPPM-6TA-L-1-F-0L6H-S1 proportional valve (6 off)

533370 VMPA-FB-EPL-G end plate

For ordering CPX Valve Terminal:

Part no: 530411

Configuration: 50E-T33GCQSYJNIJ-D, 32P-VGL-M6QXU-6QE

Configurator at:

https://www.festo.com/cat/en-gb_gb/products_VIMPA_FB?CurrentPartNo=530411

Regulator Assembly (service unit): 	Festo components as follows:

529156 MS4-LFR-1_4_D7_ERM-AS

541266 MS4-EM-1

5270709 MS4-EE-1_4-V24

For ordering MSB4 service unit, use:

Part no: 531029

Configuration: MSB4-AGC:C3:J18:D1-WP

Pneumatic Muscles:   12 off Festo DMSP 800mm 20 mm muscles (2 per actuator)

Muscle ordering details:

Part no: 541404

Configuration: DMSP-20-800N-RM-CM

Passive linear bearings for actuator rails: LUHR20-2LS SKF Linear Bearing Unit (24 off)

1000mm x 20mm (59 HRC + 6 HRC) linear shaft (3 off)

500mm x 20mm (59 HRC + 6 HRC) linear shaft (6 off)

Ball Joints	12 ball joints capable of supporting at least 250kg under tension

Ball swivel 38 degrees or greater

We used (vastly over specified) RPCSB05 from here:

https://www.steerforth.co.uk/weld-ball-socket-joint-p-4155.html

Build cost estimates (excludes VAT):

October 2019

Festo components approx.			£8k

Ball joints, linear bearings and misc. hardware	 £2k

Steel for constructing platform £		£400-£500

These costs exclude the following:

Powder coating steel components (perhaps £300)

Compressor with 6 bar capacity and at least 100-liter tank capacity (perhaps £1.5k)

Seating as required for your application

PC and display equipment as required for your application

Labor costs  for fabricating the platform

Jan 2023

Festo components approx.			£10k?

Ball joints, linear bearings and misc. hardware	 £2500

Steel for constructing platform			£650

Steel for constructing top frame			£350

These costs exclude the following:

Powder coating steel components (£700)

Compressor with 6 bar capacity and at least 100-liter tank capacity (perhaps £1.5k)

Seating as required for your application

PC and display equipment as required for your application

Labor costs  for fabricating the platform